TailorSync

Complete Body Measurement System

Production-Grade Specification for All Garment Types

| Men | Women | Kids | Traditional |
| --- | --- | --- | --- |

Full Measurement Taxonomy  •  Scan-to-Stitch Pipeline  •  Tailor Sheet Format

Database Schema  •  Accuracy Standards  •  Edge Cases & Exceptions

Prepared for: Zeeshan Niaz  •  TailorSync Platform  •  April 2026

CONFIDENTIAL — Engineering & Product Use Only

# Table of Contents

# 1. System Overview & Design Principles

TailorSync’s measurement system must produce measurements of sufficient precision that a tailor can cut and stitch a garment to an exact fit — without a single in-person fitting session. This is not a size-recommendation system. It is a precision measurement system equivalent to a professional tailor’s tape.

Non-Negotiable Accuracy Standard Every measurement delivered to a tailor must be accurate to ±0.5–1.0 cm on circumferences and ±0.3–0.5 cm on lengths. Anything outside this range requires a confidence flag and manual confirmation before the order is accepted.

## 1.1 Core Design Principles

- Complete, not approximate: The system must output every measurement a tailor needs. No partial profiles. A garment order cannot proceed until all required measurements for that garment type are present and confidence-flagged as HIGH.

- Tailor-native language: Measurement names, units (cm), and field labels must match terminology used in real tailoring shops. A tailor should be able to read the measurement sheet without training.

- Garment-specific: Each garment type has a different measurement subset. A shalwar kameez requires 18 measurements; a suit requires 24; a blouse requires 16. The system must know which measurements to request per garment.

- Confidence scoring per field: Every measurement carries a confidence score (HIGH / MEDIUM / LOW). LOW confidence fields are highlighted on the tailor sheet and flagged for manual review.

- Immutable order snapshot: When an order is placed, the full measurement set is frozen as a snapshot. Subsequent profile updates do not change existing orders.

## 1.2 Measurement Categories

| Category | Measurements Covered | Extraction Method |
| --- | --- | --- |
| Circumferences | Chest, Bust, Under-Bust, Waist, Abdomen, Hips, Neck, Bicep, Wrist, Thigh, Knee, Calf, Ankle | SMPL mesh cross-section at anatomical landmark level |
| Lengths (Vertical) | Height, Torso, Back Length, Front Length, Kameez Length, Shirt Length, Dress Length, Inseam, Outseam, Crotch Depth | Geodesic distance on mesh or landmark-to-landmark Euclidean distance |
| Widths | Shoulder Width, Chest Width, Back Width, Hip Width | Horizontal landmark-to-landmark distance on frontal projection |
| Depths (Anteroposterior) | Chest Depth, Waist Depth, Hip Depth | Side-image silhouette width at anatomical levels |
| Angles & Posture | Shoulder Slope, Back Curvature, Spine Tilt | Skeleton joint angle computation from pose landmarks |

# 2. Complete Measurement Taxonomy

The following 32 measurements form the complete TailorSync body measurement profile. All measurements are in centimetres. Each measurement is numbered for reference on tailor sheets and in the database schema.

SECTION A — UPPER BODY CIRCUMFERENCES Measurements taken around the widest point of each body zone

| # | Measurement | Definition | Scan Method | Tolerance | Garments |
| --- | --- | --- | --- | --- | --- |
| M01 | Chest / Bust | Fullest part of chest/bust. Tape horizontal, parallel to floor. | SMPL mesh circumference at chest landmark (Z-level) | ±0.8cm | All upper |
| M02 | Under-Bust | Circumference directly under the bust. Women’s garments only. | Mesh circumference 4cm below M01 Z-level | ±0.6cm | Women |
| M03 | Waist | Narrowest part of torso, typically 2–3cm above navel. | SMPL mesh minimum circumference in torso zone | ±0.6cm | All |
| M04 | Abdomen | Around the abdomen at navel level. Key for shalwar/trouser fit. | Mesh circumference at navel landmark Z-level | ±0.8cm | Pants/Shalwar |
| M05 | Hips / Seat | Fullest part of hips/buttocks. Usually 18–23cm below waist. | Mesh maximum circumference in hip zone | ±0.8cm | All lower |
| M06 | Neck | Around base of neck for collar construction. | Mesh circumference at neck base anatomical landmark | ±0.4cm | Shirts/Suits |
| M07 | Bicep | Around fullest part of upper arm, arm relaxed. | Mesh circumference at mid-upper arm Z-level | ±0.6cm | Sleeves |
| M08 | Wrist | Around wrist just above wrist bone for cuff sizing. | Mesh circumference at wrist landmark | ±0.3cm | Shirts/Suits |

SECTION B — LOWER BODY CIRCUMFERENCES Critical for trouser, shalwar, and skirt construction

| # | Measurement | Definition | Scan Method | Tolerance | Garments |
| --- | --- | --- | --- | --- | --- |
| M09 | Thigh | Fullest circumference of upper thigh, measured at crotch level. | Mesh circumference at crotch Z-level | ±0.8cm | Trousers/Shalwar |
| M10 | Mid-Thigh | Circumference at mid-point of upper leg. | Mesh circumference at 50% between crotch and knee | ±0.6cm | Fitted trousers |
| M11 | Knee | Circumference around the knee cap, leg straight. | Mesh circumference at knee landmark Z-level | ±0.6cm | Fitted trousers |
| M12 | Calf | Fullest circumference of the calf muscle. | Mesh maximum circumference in calf zone | ±0.6cm | Fitted styles |
| M13 | Ankle | Circumference just above ankle bone. | Mesh circumference at ankle landmark | ±0.4cm | Narrow hem |

SECTION C — LENGTHS & HEIGHTS Vertical measurements from landmark to landmark

| # | Measurement | Definition | Scan Method | Tolerance | Garments |
| --- | --- | --- | --- | --- | --- |
| M14 | Total Height | Floor to top of head. Used as scale reference for all other measurements. | User-input + arUco marker verification | ±0.5cm | All (reference) |
| M15 | Shoulder to Waist (Front) | Front neck base to waist, over the bust. | Landmark: front shoulder to waist landmarks (mesh path) | ±0.5cm | Blouse/Kameez |
| M16 | Shoulder to Waist (Back) | Back neck base to waist, straight down. | Landmark: back neck to waist on mesh | ±0.5cm | All upper |
| M17 | Kameez / Shirt Length | Back neck to desired hem. Varies by garment style. | User-defined hem level on avatar (adjustable slider) | ±0.5cm | Kameez/Shirts |
| M18 | Dress / Suit Length | Shoulder to hem for dresses; jacket hem for suits. | User-defined hem level on avatar | ±0.5cm | Dresses/Suits |
| M19 | Sleeve Length | From shoulder point to wrist bone, arm slightly bent. | Landmark: shoulder to wrist, geodesic on mesh | ±0.5cm | All with sleeves |
| M20 | Sleeve Length (Elbow) | Shoulder to elbow for 3/4 or elbow-length sleeves. | Landmark: shoulder to elbow landmark | ±0.5cm | Optional styles |
| M21 | Inseam | Crotch to floor, inside leg. Used for trouser inseam length. | Landmark: crotch to ankle on inner leg path (mesh) | ±0.5cm | Trousers/Shalwar |
| M22 | Outseam | Waist to floor, outside leg. Full trouser side length. | Landmark: waist side to ankle on outer leg path | ±0.5cm | Trousers |
| M23 | Crotch Depth (Front) | Waist to crotch, sitting. Determines rise of trousers. | Side silhouette: waist to crotch line depth | ±0.5cm | Trousers/Shalwar |
| M24 | Crotch Depth (Back) | Back waist to crotch, determines back rise. | Side silhouette + mesh computation | ±0.5cm | Trousers/Shalwar |
| M25 | Torso Length | Neck to crotch, full torso. Used for bodysuits and kameez proportion. | Landmark: neck base to crotch on front mesh path | ±0.6cm | Kameez/Bodysuit |

SECTION D — WIDTHS & DEPTHS Horizontal and depth measurements for garment structure

| # | Measurement | Definition | Scan Method | Tolerance | Garments |
| --- | --- | --- | --- | --- | --- |
| M26 | Shoulder Width | Tip of one shoulder to tip of the other, across back. | Landmark: left to right shoulder point, rear projection | ±0.5cm | All upper |
| M27 | Chest Width | Across the chest from armhole to armhole (not full circumference). | Landmark: left to right armhole at chest level, front only | ±0.5cm | Shirts/Suits |
| M28 | Back Width | Across the back between armholes at shoulder blade level. | Landmark: left to right armhole, rear projection | ±0.5cm | Shirts/Suits/Kameez |
| M29 | Hip Width | Widest point of hips, measured flat (not circumference). | Landmark: left to right hip point, frontal projection | ±0.5cm | Skirts/Dresses |
| M30 | Chest Depth | Front-to-back thickness at chest level. Derived from side image. | Side silhouette width at chest Z-level | ±0.8cm | 3D avatar only |
| M31 | Waist Depth | Front-to-back thickness at waist. Detected from side image. | Side silhouette width at waist Z-level | ±0.8cm | 3D avatar only |
| M32 | Armhole Depth | Vertical depth of armhole from shoulder seam to armhole bottom. | Landmark: shoulder to armhole bottom on mesh | ±0.4cm | Jackets/Suits |

Measurement Completeness Rule An order for any garment type CANNOT be submitted unless all Required measurements for that garment show confidence HIGH or MEDIUM. LOW confidence fields must be manually confirmed or re-scanned before checkout proceeds.

# 3. Garment-Type Measurement Mapping

Each garment type requires a specific subset of the 32 measurements. The table below defines, for every supported garment type, which measurements are Required (R), Optional (O), or Not Applicable (—).

## 3.1 Men’s Garments

| Measurement | Shalwar Kameez | Dress Shirt | Suit Jacket | Trousers | Waistcoat | Sherwani | Kurta Pajama |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M01 Chest | R | R | R | — | R | R | R |
| M02 Under-Bust | — | — | — | — | — | — | — |
| M03 Waist | R | R | R | R | R | R | R |
| M04 Abdomen | R | — | — | R | — | R | R |
| M05 Hips | R | — | — | R | — | R | R |
| M06 Neck | R | R | R | — | R | R | R |
| M07 Bicep | O | O | R | — | — | R | O |
| M08 Wrist | O | R | R | — | — | R | O |
| M09 Thigh | R | — | — | R | — | O | R |
| M10 Mid-Thigh | — | — | — | O | — | — | — |
| M11 Knee | O | — | — | O | — | O | O |
| M13 Ankle | R | — | — | O | — | R | R |
| M14 Height | R | R | R | R | R | R | R |
| M15 Shoulder–Waist (F) | R | R | R | — | R | R | R |
| M16 Shoulder–Waist (B) | R | R | R | — | R | R | R |
| M17 Kameez/Shirt Length | R | R | — | — | — | R | R |
| M18 Jacket Length | — | — | R | — | R | — | — |
| M19 Sleeve Length | R | R | R | — | O | R | R |
| M21 Inseam | R | — | — | R | — | R | R |
| M22 Outseam | O | — | — | R | — | O | O |
| M23 Crotch (Front) | R | — | — | R | — | R | R |
| M24 Crotch (Back) | R | — | — | R | — | R | R |
| M26 Shoulder Width | R | R | R | — | R | R | R |
| M27 Chest Width | — | R | R | — | R | R | — |
| M28 Back Width | R | R | R | — | R | R | R |
| M32 Armhole Depth | — | O | R | — | R | — | O |

## 3.2 Women’s Garments

| Measurement | Shalwar Kameez | Anarkali | Lehenga Blouse | Western Dress | Saree Blouse | Abaya | Palazzo Suit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M01 Bust | R | R | R | R | R | R | R |
| M02 Under-Bust | R | R | R | R | R | O | R |
| M03 Waist | R | R | R | R | R | R | R |
| M04 Abdomen | O | O | — | O | — | R | O |
| M05 Hips | R | R | R | R | R | R | R |
| M06 Neck | R | R | R | R | R | R | R |
| M07 Bicep | O | O | R | O | R | O | O |
| M08 Wrist | O | O | R | O | R | O | O |
| M09 Thigh | R | O | — | O | — | R | R |
| M13 Ankle | R | O | — | O | — | R | R |
| M14 Height | R | R | R | R | R | R | R |
| M15 Shoulder–Waist (F) | R | R | R | R | R | R | R |
| M16 Shoulder–Waist (B) | R | R | R | R | R | R | R |
| M17 Kameez Length | R | R | — | — | — | R | R |
| M18 Dress Length | — | — | — | R | — | — | — |
| M19 Sleeve Length | R | R | R | O | R | R | R |
| M20 Sleeve to Elbow | O | O | R | O | O | O | O |
| M21 Inseam | R | — | — | — | — | O | R |
| M23 Crotch (Front) | R | — | — | — | — | O | R |
| M26 Shoulder Width | R | R | R | R | R | R | R |
| M27 Chest Width | — | R | R | R | R | R | — |
| M28 Back Width | R | R | R | R | R | R | R |
| M29 Hip Width | — | R | R | R | — | R | — |
| M25 Torso Length | O | R | O | R | O | R | O |

## 3.3 Children’s Garments

| Measurement | Kids Kameez | School Uniform | Kids Suit | Kids Dress |
| --- | --- | --- | --- | --- |
| M01 Chest | R | R | R | R |
| M03 Waist | R | R | R | R |
| M05 Hips | R | R | R | R |
| M06 Neck | O | R | R | O |
| M14 Height | R | R | R | R |
| M16 Back Length | R | R | R | R |
| M17 Shirt Length | R | R | — | — |
| M18 Dress Length | — | — | — | R |
| M19 Sleeve Length | R | R | R | O |
| M21 Inseam | R | R | R | — |
| M26 Shoulder Width | R | R | R | R |

# 4. Complete Scan-to-Measurement Pipeline

The pipeline has five sequential stages. Each stage has defined inputs, outputs, failure modes, and fallback actions.

## Stage 1 — Guided Capture (On-Device)

| Step | Action | Tool / Library | Pass Criteria | Failure Action |
| --- | --- | --- | --- | --- |
| 1.1 | Environment check | OpenCV brightness analysis (on-device) | 100–250 lux ambient; no direct backlight | Show ‘Move to better lighting’ prompt |
| 1.2 | Camera position guide | AR overlay (ar_flutter_plugin) | Camera 2.0–2.5m from subject; 0°–5° tilt | Show distance indicator and leveling guide |
| 1.3 | Background quality | MediaPipe Selfie Segmentation | Contrast score > 0.6 between body and background | Suggest plain wall / different background |
| 1.4 | Real-time pose overlay | google_mlkit_pose_detection (30fps) | Show ghost silhouette target for front & side poses | Continuous feedback: arrows and text instructions |
| 1.5 | Pose lock & capture | ML Kit pose alignment score | Score ≥ 87% for 1.5 continuous seconds | Do not allow manual capture until score met |
| 1.6 | Image quality check | OpenCV blur detection (Laplacian variance) | Variance > 120 (not blurry) | Auto-retake if blurry |

## Stage 2 — Pre-processing (Server)

| Step | Action | Tool | Output |
| --- | --- | --- | --- |
| 2.1 | EXIF strip + normalize | Pillow (Python) | Orientation-corrected, EXIF-clean image |
| 2.2 | Resize to 1280×1280px | OpenCV | Consistent input resolution for models |
| 2.3 | Body segmentation | MediaPipe Selfie Segmentation | Binary mask: body vs. background |
| 2.4 | Masked image generation | OpenCV + mask | Clean body silhouette on white background |
| 2.5 | Silhouette contour extraction | OpenCV findContours | Ordered point array of body outline |
| 2.6 | Quality gate | Custom validation | Reject if body < 40% of frame; prompt re-scan |

## Stage 3 — Pose Estimation & Landmark Extraction (Server)

| Step | Action | Model | Output | Landmarks Used |
| --- | --- | --- | --- | --- |
| 3.1 | 33-point pose estimation (frontal) | MediaPipe BlazePose (server) | 3D keypoints (x,y,z) + visibility score | All 33 points |
| 3.2 | 33-point pose estimation (side) | MediaPipe BlazePose (server) | 3D keypoints (x,y,z) + visibility score | Lateral profile points |
| 3.3 | Landmark visibility filter | Custom | Flag low-visibility landmarks for confidence scoring | Per-landmark score |
| 3.4 | SMPL body shape fitting | PyMAF-X or 4D-Humans | SMPL β (10 shape params) + θ (72 pose params) | Full mesh of 6,890 vertices |
| 3.5 | Mesh alignment verification | Custom ICP alignment | Confirm mesh aligns with silhouette outline ≥ 92% | Global fit score |

## Stage 4 — Measurement Extraction from SMPL Mesh (Server)

Once the SMPL body mesh is fitted, all 32 measurements are computed analytically. This is the critical stage that determines accuracy.

### 4.1 Circumference Extraction Method

For each circumference measurement (M01–M13):

- Identify the anatomical Z-level on the SMPL mesh using the corresponding body landmark (e.g., chest circumference at the Z-coordinate of the armpits)

- Slice the mesh with a horizontal plane at that Z-level

- Extract the closed contour of that cross-section

- Compute perimeter of the contour = circumference measurement

- Apply a clothing allowance offset (see Section 5.3 for values per measurement)

### 4.2 Length Extraction Method

For each length measurement (M14–M25):

- Identify start and end anatomical landmarks on the SMPL mesh

- Compute the geodesic path (surface distance) between the two points along the mesh

- Geodesic distance = measurement length

- For straight lengths (e.g., inseam), use Euclidean distance with gravity correction

### 4.3 Width Extraction Method

For each width measurement (M26–M32):

- Project the SMPL mesh onto the frontal plane (XY)

- Identify left and right landmark points at the corresponding body level

- Euclidean distance between the two projected points = width

## Stage 5 — Confidence Scoring & Output

| Confidence Level | Criteria | Action on Tailor Sheet | Action on Order Flow |
| --- | --- | --- | --- |
| HIGH ✅ | SMPL mesh fit ≥ 92%; landmark visibility ≥ 0.85; within 2σ of population distribution | Green value; no flag | Order proceeds normally |
| MEDIUM ⚠️ | Mesh fit 80–92% OR landmark visibility 0.65–0.85 OR mild outlier (2–3σ) | Yellow highlight; review suggested | Tailor notified; can proceed with acknowledgement |
| LOW ❌ | Mesh fit < 80% OR visibility < 0.65 OR statistical outlier > 3σ | Red flag; manual input required | Order BLOCKED until measurement confirmed or re-scanned |

# 5. Accuracy Engineering

## 5.1 Scale Calibration — The Most Critical Factor

Without accurate scale, all circumferences and lengths will be off by the same proportional error. TailorSync uses a three-tier scale calibration hierarchy:

| Tier | Method | Accuracy | User Effort | Use Case |
| --- | --- | --- | --- | --- |
| Tier 1 (Default) | User inputs height; system computes pixels-per-cm from leg landmark span | ±1.5cm | Low — one field | All users |
| Tier 2 (Better) | Standard door frame detected in image (203cm reference); auto-calibrates scale | ±0.8cm | Zero — automatic | User stands near a door |
| Tier 3 (Best) | Printed QR reference card (20×20cm) held at waist; computer vision reads exact size | ±0.3cm | Medium — print & hold | Premium users / high-value orders |

## 5.2 Clothing Compensation

Scans are taken with the user wearing form-fitting clothing (required by the app). However, even tight clothing adds volume to measurements. Apply these offsets:

| Measurement | Clothing Type | Offset to Subtract | Rationale |
| --- | --- | --- | --- |
| Chest / Bust | T-shirt / fitted top | −0.8 cm | Fabric + bra padding |
| Waist | Leggings / fitted | −0.4 cm | Minimal fabric |
| Hips | Leggings | −0.6 cm | Fabric layer |
| Thigh | Leggings | −0.5 cm | Fabric layer |
| Bicep | T-shirt | −0.4 cm | Sleeve fabric |
| Shoulders | Any top | −0.3 cm | Seam and fabric |
| Neck | Any top | −0.2 cm | Minimal at base |
| Inseam / Lengths | Any | +0.0 cm | Length not affected by clothing |

## 5.3 Tailoring Ease Allowances

Raw body measurements are not the same as garment measurements. Tailors add “ease” for movement and comfort. The TailorSync system outputs both: the raw body measurement AND the recommended stitching measurement. The tailor receives the stitching measurement.

| Measurement | Fitted (cm) | Regular (cm) | Relaxed (cm) | Notes |
| --- | --- | --- | --- | --- |
| Chest / Bust | +5–6 | +8–10 | +12–14 | Varies by garment style selection |
| Waist | +3–4 | +5–6 | +8–10 | Kameez waist has more ease |
| Hips | +5–6 | +8–10 | +12–16 | Shalwar has more ease |
| Seat (Shalwar) | +8–10 | +12–15 | +18–22 | Traditional shalwar = very relaxed |
| Shoulder | +0.5 | +1.0 | +1.5 | Minor ease only |
| Sleeve | +1.5–2 | +2–3 | +3–4 | Movement allowance |
| Neck | +0.5 | +1.0 | +1.5 | Collar ease |

Ease Selection The customer selects the fit style (Fitted / Regular / Relaxed) on the order screen. This selection is applied globally to all ease allowances. The garment-specific measurements are computed accordingly and transmitted to the tailor.

# 6. Tailor Measurement Sheet Specification

The tailor receives a structured measurement sheet for every order. It contains two sections: customer measurements (body) and stitching measurements (body + ease). The tailor uses the stitching measurements exclusively for cutting fabric.

## 6.1 Sheet Structure

| Section | Content | Who Uses It |
| --- | --- | --- |
| Order Header | Order ID, customer name, garment type, fabric, style reference image, delivery date | Tailor — order identification |
| Fit Style | Fitted / Regular / Relaxed selection + customer notes | Tailor — determines ease application |
| Body Measurements | Raw scan measurements (32 fields), confidence flags, unit: cm | Tailor reference / QA check |
| Stitching Measurements | Body + ease for each required field. Bold, large font. These are the CUT measurements | Tailor — primary cutting reference |
| Special Instructions | Customer free-text notes (e.g., ‘extra length on sleeves’, ‘loose at hips’) | Tailor — custom adjustments |
| Confidence Summary | List of any LOW/MEDIUM confidence measurements with recommended manual verification | Tailor — quality gate |

## 6.2 Sample Stitching Measurement Block (Shalwar Kameez — Men)

| # | Measurement | Body (cm) | Ease (cm) | Stitch (cm) | Confidence |
| --- | --- | --- | --- | --- | --- |
| M01 | Chest | 96.0 | +10 | 106.0 | ✅ HIGH |
| M03 | Waist | 82.0 | +6 | 88.0 | ✅ HIGH |
| M04 | Abdomen | 88.0 | +8 | 96.0 | ✅ HIGH |
| M05 | Hips / Seat | 98.0 | +14 | 112.0 | ✅ HIGH |
| M06 | Neck | 38.5 | +1 | 39.5 | ✅ HIGH |
| M09 | Thigh | 58.0 | +12 | 70.0 | ⚠️ MED |
| M13 | Ankle | 26.0 | +2 | 28.0 | ✅ HIGH |
| M17 | Kameez Length | 72.0 | +0 | 72.0 | ✅ HIGH |
| M19 | Sleeve Length | 61.0 | +3 | 64.0 | ✅ HIGH |
| M21 | Inseam | 79.0 | +0 | 79.0 | ✅ HIGH |
| M23 | Crotch Depth (F) | 27.5 | +2 | 29.5 | ✅ HIGH |
| M24 | Crotch Depth (B) | 33.0 | +3 | 36.0 | ⚠️ MED |
| M26 | Shoulder Width | 44.0 | +1 | 45.0 | ✅ HIGH |
| M28 | Back Width | 38.5 | +2 | 40.5 | ✅ HIGH |

Tailor Note Format Confidence MEDIUM on M09 Thigh: scan showed slight occlusion on right thigh. Recommended action: verify against customer’s last order or request manual measurement. Order can proceed if tailor acknowledges.

# 7. Database Schema

## 7.1 Core Tables

### measurement_profiles

| Column | Type | Description |
| --- | --- | --- |
| id | UUID PRIMARY KEY | Unique profile ID |
| user_id | UUID FK → users | Owner of the profile |
| scan_date | TIMESTAMPTZ | When the scan was performed |
| scan_method | ENUM(TWO_PHOTO, VIDEO, LIDAR) | Capture method used |
| height_cm | DECIMAL(5,1) | User-input height for scale reference |
| scale_tier | ENUM(TIER1, TIER2, TIER3) | Calibration quality used |
| smpl_beta | FLOAT[10] | SMPL shape parameters (stored for avatar regeneration) |
| smpl_theta | FLOAT[72] | SMPL pose parameters |
| overall_confidence | ENUM(HIGH, MEDIUM, LOW) | Lowest confidence across all required fields |
| is_active | BOOLEAN DEFAULT TRUE | Only one active profile per user |
| created_at | TIMESTAMPTZ | Record creation time |

### measurements

| Column | Type | Description |
| --- | --- | --- |
| id | UUID PRIMARY KEY | Unique measurement row |
| profile_id | UUID FK → profiles | Parent scan profile |
| code | VARCHAR(4) | e.g. M01, M17 (matches taxonomy) |
| name | VARCHAR(80) | Human-readable name |
| body_value_cm | DECIMAL(5,2) | Raw scan measurement in cm |
| confidence | ENUM(HIGH, MEDIUM, LOW) | Per-field confidence score |
| mesh_fit_score | DECIMAL(4,3) | SMPL mesh fit quality for this measurement |
| landmark_visibility | DECIMAL(4,3) | Average landmark visibility score |
| is_manual_override | BOOLEAN | True if user or tailor manually set this value |
| override_value_cm | DECIMAL(5,2) NULL | Manual override value, if set |
| notes | TEXT NULL | Tailor or system notes for this field |

### order_measurement_snapshots

| Column | Type | Description |
| --- | --- | --- |
| id | UUID PRIMARY KEY | Snapshot ID |
| order_id | UUID FK → orders | Order this snapshot belongs to |
| profile_id | UUID FK → profiles | Source profile |
| garment_type | VARCHAR(60) | e.g. SHALWAR_KAMEEZ_MEN |
| fit_style | ENUM(FITTED, REGULAR, RELAXED) | Customer’s chosen ease profile |
| measurements_json | JSONB | Full frozen snapshot: { code, body_cm, ease_cm, stitch_cm, confidence }[] |
| special_notes | TEXT NULL | Customer free-text at order time |
| tailor_confirmed | BOOLEAN DEFAULT FALSE | Tailor acknowledged low-confidence flags |
| snapshot_at | TIMESTAMPTZ | Immutable — set at order placement |

# 8. Edge Cases & Exception Handling

| Scenario | Detection Method | System Response |
| --- | --- | --- |
| Pregnant user | Abdomen circumference outlier (>3σ above hip-waist ratio normal) | Flag measurements; show advisory: ‘Some measurements may need adjustment. Please add a note to your order.’ |
| Very large body type (BMI > 40) | Total body width vs. height ratio | Switch to relaxed ease profile automatically; flag for tailor review |
| User wearing hijab/scarf | Head-to-shoulder landmark distance anomaly | Neck and shoulder measurements flagged LOW; prompt: ‘Please remove headwear or enter neck/shoulder manually’ |
| User with prosthetic limb | Asymmetric arm/leg landmarks | Detect asymmetry; prompt user to specify affected side; only measure the healthy side; note to tailor |
| Child (height < 120cm) | Height input trigger | Switch to children’s measurement schema; reduced measurement set |
| Poor lighting (night scan) | Brightness < 80 lux | Block scan attempt; prompt ‘Please scan in daylight or under bright indoor light’ |
| Full-body covering (abaya/niqab) | Segmentation low contrast | Alert: ‘For accurate measurements, please scan in form-fitting clothing. Alternatively, enter measurements manually.’ |
| Camera too close (< 1.5m) | Pose landmark bounding box > 80% of frame | Show ‘Step back’ AR indicator; block capture |
| Measurement outlier post-scan | Any value outside 3σ of population model | Automatically flag LOW confidence; show ‘This measurement looks unusual — please verify’ |
| Repeat scan with large delta | Compare new scan to existing profile; delta > 5cm on any field | Alert: ‘This scan differs significantly from your last profile. Would you like to update your profile?’ |

# 9. Customer Scan UX Flow

The scan experience must be guided, confidence-building, and fast. Target total scan time: under 3 minutes including instructional screens.

| Screen | Content | User Action | Duration |
| --- | --- | --- | --- |
| 1. Intro | What we’re measuring and why. Privacy statement: images deleted after 30 days. | Tap ‘Start Scan’ | 15–20s |
| 2. Clothing guide | Illustrated guide: wear fitted clothing (t-shirt + leggings/trousers). No scarves, jackets, or loose layers. | Tap ‘I’m Ready’ | 10–15s |
| 3. Height input | Enter height in cm or ft/in. Used for scale calibration. | Enter height; tap ‘Continue’ | 10s |
| 4. Environment setup | Real-time camera with lighting and distance indicators. Green = ready. | Move to correct position | 30–60s |
| 5. Front scan | Camera with real-time skeleton overlay. Ghost silhouette shows target pose. Score indicator. | Hold pose until auto-capture fires at 87%+ | 30–60s |
| 6. Side scan | Same flow, now rotated 90°. Ghost silhouette for side profile. | Hold pose until auto-capture fires | 30–60s |
| 7. Processing | Animated progress: ‘Analysing your body shape… Extracting measurements… Finalising profile…’ | Wait | 5–15s |
| 8. Results | Show all measurements with confidence flags. Allow manual editing of any field. | Review + confirm or edit | 30–60s |
| 9. Profile saved | Avatar preview (Phase 2). Measurements saved. Ready to order. | Tap ‘Browse Collections’ | 5s |

# 10. Quality Assurance & Continuous Improvement

## 10.1 Post-Delivery Feedback Loop

After each garment is delivered, TailorSync collects a structured fit rating from the customer. This data is the primary driver of measurement accuracy improvement.

| Feedback Field | Type | Use in System |
| --- | --- | --- |
| Overall fit rating | 1–5 stars | Global accuracy metric (target avg > 4.2) |
| Per-measurement delta | Number entry (cm) | Train correction model per measurement per body type |
| Problem area | Multi-select chips | Tag: ‘chest too tight’, ‘waist too loose’, etc. |
| Photos of garment | Optional upload | Computer vision analysis of fit issues (Phase 3) |
| Would re-order? | Yes / No | Business retention metric |

## 10.2 Accuracy Benchmarks

| Metric | Phase 1 Target | Phase 2 Target | Industry Best |
| --- | --- | --- | --- |
| Mean Absolute Error (chest/waist/hips) | ±2.0 cm | ±1.2 cm | ±0.8 cm (MTailor) |
| Mean Absolute Error (lengths) | ±1.5 cm | ±0.8 cm | ±0.5 cm |
| Scan success rate (no re-scan needed) | 75% | 88% | 92% (MTailor) |
| Customer fit rating after delivery | > 3.8 / 5 | > 4.2 / 5 | > 4.5 / 5 |
| Orders requiring alteration after delivery | < 20% | < 10% | < 5% |

# 11. Complete Technical Stack

| Layer | Component | Technology | Justification |
| --- | --- | --- | --- |
| Mobile | Camera capture | Flutter camera package | Full exposure/focus control |
| Mobile | Real-time pose guidance | google_mlkit_pose_detection | On-device, 30fps, no latency |
| Mobile | Segmentation (on-device) | MediaPipe Selfie Segmentation | Real-time background removal |
| Mobile | Avatar display | flutter_3d_controller (.glb) | Lightweight 3D render |
| Mobile | AR calibration overlay | ar_flutter_plugin (ARCore/ARKit) | Distance & angle guides |
| Backend API | REST framework | FastAPI (Python 3.11) | Async, fast, auto-docs |
| Backend API | Task queue | Celery + Redis | Queue heavy GPU jobs; async response |
| AI — Pose | Server pose extraction | MediaPipe BlazePose (Python) | High-res landmark accuracy |
| AI — Segment | Body segmentation | MediaPipe Selfie + SAM fallback | SAM for difficult cases |
| AI — 3D Body | SMPL fitting (Phase 1) | PyMAF-X | Best single-image SMPL fit |
| AI — 3D Body | SMPL fitting (Phase 2) | 4D-Humans / HMR2.0 | Transformer accuracy upgrade |
| AI — Measurements | Mesh measurement extraction | Custom Python + Open3D | Geodesic computation on mesh |
| AI — Try-On | 2D virtual try-on | Fashn.ai API / IDM-VTON | State-of-art 2024 try-on |
| AI — Avatar | 3D avatar generation | Ready Player Me API | Fast; Flutter SDK available |
| Database | Measurements store | PostgreSQL 15 | Relational + JSONB for snapshots |
| Storage | Scan images | AWS S3 (30-day auto-delete) | Privacy compliant |
| Storage | Avatar .glb files | Cloudflare R2 + CDN | Fast global delivery |
| Storage | Cache (try-on images) | Redis (24h TTL) | Avoid duplicate API calls |
| Infra | GPU inference server | AWS g4dn.xlarge (NVIDIA T4) | PyMAF-X inference |
| Infra | Monitoring | Sentry + Datadog | Error tracking + accuracy metrics |

# 12. Summary — The Complete Solution

TailorSync’s measurement system is designed to replace the tailor’s tape entirely. The architecture below summarises the full end-to-end flow from customer scan to tailor-ready measurement sheet:

| Stage | What Happens | Output |
| --- | --- | --- |
| Customer scans | Guided 2-photo capture with real-time pose scoring, lighting check, and auto-capture on pose lock | Two high-quality images + height input |
| Pre-processing | Segmentation, masking, silhouette extraction, quality gate | Clean body silhouette ready for AI |
| AI body fitting | MediaPipe landmarks + PyMAF-X SMPL mesh fitting | 3D body mesh (6,890 vertices) with shape params |
| Measurement extraction | Geodesic circumferences and lengths from SMPL mesh + confidence scoring | 32 measurements with HIGH/MEDIUM/LOW confidence |
| Ease computation | Apply fit style (Fitted/Regular/Relaxed) ease table per garment type | Stitching measurements per garment |
| Tailor sheet generated | PDF/digital sheet: body measurements + stitching measurements + confidence flags + customer notes | Tailor receives everything needed to cut fabric |
| Order placed | Measurement snapshot frozen; order enters production queue | Immutable order record for traceability |
| Post-delivery loop | Customer rates fit; delta data collected; model fine-tuned per body type | Continuously improving accuracy over time |

Bottom Line The system outputs 32 precision measurements per customer, mapped to the exact fields each garment type requires, with confidence scoring on every field, ease allowances computed by fit style, and a tailor-native measurement sheet that a stitching professional can act on immediately — no in-person fitting session, no phone calls, no guesswork.

— End of Document —