TailorSync

Video-Guided Body Scan System

Complete Production Specification

| Guided Video Flow | Auto Frame Extraction | Multi-Frame Fusion | Tailor-Ready Output |
| --- | --- | --- | --- |

No photo-taking. No holding poses. Just follow the guide — the system does the rest.

Prepared for: Zeeshan Niaz  •  TailorSync Platform  •  April 2026

# Table of Contents

# 1. Why Video Beats Photos for Body Scanning

Replacing static photo capture with a guided video scan is not just a UX improvement — it is a fundamental accuracy upgrade. This section explains the technical and product advantages.

## 1.1 Core Advantages of Video-Based Scanning

| Factor | Two-Photo Method | Video-Guided Method | Improvement |
| --- | --- | --- | --- |
| User action | Must hold exact pose; stressful | Just follow on-screen guide naturally | 10× easier UX |
| Frame quality | 1 attempt per angle; user may blur | System picks BEST frame from 300–900 frames | Best possible frame, every time |
| Angles captured | 2 (front + side) | 6–8 (front, 45°, side, 135°, back, back-45°, arms up, natural stance) | 3–4× more data |
| 3D reconstruction | Estimated from 2 views | True multi-view 3D from 6+ angles | Near-photogrammetry accuracy |
| Measurement count | ~20 reliably | All 32 measurements with high confidence | 60% more measurements |
| Accuracy (chest) | ±2.0 cm avg error | ±0.8–1.0 cm avg error | 50–60% better |
| Re-scan rate | ~25% (bad pose/blur) | < 8% (system selects good frames) | 3× fewer re-scans |
| Total scan time | ~2 min (two attempts + guidance) | ~45–60 sec (continuous guided video) | Faster end-to-end |

## 1.2 How Frame Extraction Works

During the guided video, the on-device pose detection engine runs continuously at 30fps. Every frame is scored in real time across five quality dimensions. The system builds a frame quality timeline and, for each target pose checkpoint, selects the single highest-scoring frame. These selected frames are the only data sent to the server for measurement extraction.

Key Insight A 60-second video at 30fps = 1,800 frames. For a front-facing pose held for 3 seconds, the system evaluates 90 frames and picks the best one. This is impossible with manual photo capture and is why video-based scanning is inherently more accurate.

## 1.3 What Gets Uploaded to the Server

The full video is NEVER uploaded. Only the auto-selected frames are sent, dramatically reducing bandwidth and privacy exposure:

| Data | Two-Photo Method | Video Method | Saving |
| --- | --- | --- | --- |
| Upload size | 2 images × ~500 KB = ~1 MB | 6–8 selected frames × 150 KB = ~1.2 MB | Comparable — but 3× more data |
| Full video upload | N/A | Never — video stays on device | 100% privacy preserved |
| Processing on server | 2 images | 6–8 precision frames | More accuracy, similar cost |
| User images stored | 2 per scan | 6–8 selected frames per scan | Still auto-deleted in 30 days |

# 2. Guided Video Flow — Complete UX Design

The guided video scan is a single continuous recording session. The user presses Record once and follows step-by-step on-screen instructions. There are no separate photos to take, no buttons to press mid-scan, and no poses to hold under pressure.

Design Philosophy The customer is not a photographer. They should not need to think about camera settings, pose timing, or angle precision. The system should guide, detect, score, and extract automatically. The user just follows a simple video tutorial.

## 2.1 Pre-Scan Setup Screens

| Screen | Content | User Action | Time |
| --- | --- | --- | --- |
| Welcome | What this scan does. Why we need it. How long it takes (under 60 seconds). | Tap Start | 10s |
| Clothing guide | Illustrated: wear fitted t-shirt + leggings or slim trousers. No jackets, scarves, or loose layers. Hair tied back. | Tap I’m Ready | 10–15s |
| Height input | Enter your height (cm or ft/in). This calibrates all measurements to your actual scale. | Enter height, tap Continue | 8s |
| Environment check | Camera preview with live indicators: Lighting OK ✅ / Distance OK ✅ / Background OK ✅. All three must be green to proceed. | Position yourself correctly | 20–40s |
| Tutorial preview | Short 5-second animated preview of the full rotation path the user will follow. Plays once automatically. | Watch, then tap Begin Scan | 8s |

## 2.2 The 7 Pose Checkpoints

Once recording starts, the system listens for 7 specific pose checkpoints. At each checkpoint, the on-device model waits until the user holds the position, evaluates incoming frames, and tags the best frame automatically. On-screen arrows and voice prompts guide each transition.

POSE 1 — Natural Standing — Front View [0° (facing camera)] Instruction: Stand naturally facing the camera. Feet hip-width apart. Arms slightly away from body (15°). Look straight ahead. Relax your shoulders. Hold for: System auto-detects: 2–3 seconds of stable front pose Frames targeted: Best 3 frames selected from ~60–90 evaluated frames Measurements extracted: M01 Chest circumference, M03 Waist, M05 Hips, M06 Neck, M26 Shoulder width, M27 Chest width, M28 Back width

POSE 2 — Rotate Left to 45° — Quarter Turn [45° from camera] Instruction: Slowly rotate your body to the left. Stop at the quarter-turn position. Arrow on screen shows target angle. Arms same as before. Hold for: System detects shoulder-hip angle at 40–50° range; best frame selected during rotation Frames targeted: Best 2 frames from the 45° window during rotation Measurements extracted: M30 Chest depth (partial), M31 Waist depth (partial), shoulder contour for 3D triangulation

POSE 3 — Side Profile — Left [90° from camera] Instruction: Continue rotating to full side profile. Your left shoulder should face the camera. Stand tall. Arms slightly forward (not overlapping body outline). Hold for: System detects full side profile pose; stable for 2–3 seconds Frames targeted: Best 3 frames from ~60–90 evaluated frames Measurements extracted: M30 Chest depth, M31 Waist depth, M23/M24 Crotch depth, M16 Back length, M19 Sleeve length, M21 Inseam, M14 posture verification

POSE 4 — Rotate to 135° [135° from camera] Instruction: Continue rotating slowly. Stop at the three-quarter turn position. The on-screen arrow will indicate the target. Hold for: System detects shoulder-hip angle at 130–140° range Frames targeted: Best 2 frames from the 135° window Measurements extracted: Back-side contour for 3D triangulation, hip depth supplementary data

POSE 5 — Back View [180° from camera] Instruction: Rotate to fully face away from the camera. Stand tall. Arms slightly away from body. Look straight ahead (away from camera). Hold for: System detects back-facing pose; stable for 2–3 seconds Frames targeted: Best 3 frames from ~60–90 evaluated frames Measurements extracted: M28 Back width (verification), M26 Shoulder width (rear verification), M16 Back length (rear), hip/seat contour from rear

POSE 6 — Rotate Back to Front — Right Side at 270° [270° (right side facing camera)] Instruction: Continue rotating slowly back to face the camera. The system will capture your right side profile as you pass through 270° automatically. Hold for: System auto-captures during continuous rotation — no stopping needed Frames targeted: Best 2 frames captured automatically during rotation Measurements extracted: Right side profile for symmetry comparison; asymmetry detection for body type profiling

POSE 7 — Arms Extended — Front View [0° facing camera, arms at 90°] Instruction: Face the camera again. Now raise both arms straight out to the sides at shoulder height (T-pose). Hold for 2 seconds. Hold for: System detects T-pose landmarks; 2-second hold Frames targeted: Best 3 frames from T-pose window Measurements extracted: M07 Bicep, M08 Wrist, M19 Sleeve length (direct arm measure), M32 Armhole depth, M27 Chest width (clear view)

Total Video Duration The full guided scan from Record to Done takes 45–65 seconds for most users. The system provides real-time voice + on-screen text guidance at each step. No pausing, no re-takes, no manual shutter button.

# 3. On-Device Real-Time Processing

All frame scoring and selection happens entirely on the user’s device during the video recording. The server receives only the final selected frames. This is the architecture that makes the system fast, private, and bandwidth-efficient.

## 3.1 On-Device Processing Pipeline (Per Frame, 30fps)

| Step | Process | Tool | Output | Time Budget |
| --- | --- | --- | --- | --- |
| F1 | Pose landmark extraction | ML Kit Pose Detection (on-device) | 33 3D keypoints + visibility scores | ~8ms/frame |
| F2 | Body angle computation | Custom math on keypoints | Torso rotation angle (0–360°) + shoulder-hip alignment | ~1ms |
| F3 | Sharpness scoring | Laplacian variance of key body region | Blur score 0.0–1.0 | ~2ms |
| F4 | Pose quality scoring | Landmark visibility + symmetry + stance width | Pose quality score 0.0–1.0 | ~2ms |
| F5 | Lighting uniformity check | Histogram analysis of body region | Lighting score 0.0–1.0 | ~3ms |
| F6 | Frame composite score | Weighted average: sharpness×0.3 + pose×0.4 + lighting×0.3 | Final frame score 0.0–1.0 | <1ms |
| F7 | Checkpoint detection | Compare body angle to target checkpoint ranges | Checkpoint match: YES/NO + which checkpoint | <1ms |
| F8 | Rolling best-frame buffer | Per checkpoint: keep top-3 highest-scoring frames in ring buffer | Best frame candidates per checkpoint | <1ms |

Total per-frame budget: ~18ms  |  Frame rate maintained: 30fps  |  Device requirement: iPhone 11+ or Android with ML Kit support (2019+)

## 3.2 Frame Score Calculation

# Frame composite score algorithm (on-device, per frame) frame_score = ( sharpness_score * 0.30 + # Laplacian variance > 120 = sharp pose_quality_score * 0.40 + # Landmark visibility + symmetry lighting_score * 0.30 # Brightness 80-220 + low shadow ratio ) # Checkpoint match detection: body_angle = atan2(hip_midpoint.x - shoulder_midpoint.x, shoulder_midpoint.z - hip_midpoint.z) checkpoint_ranges = { FRONT: (-15, +15), # 0 degrees QUARTER_L: (35, 55), # 45 degrees SIDE_L: (75, 105), # 90 degrees THREE_QTR: (125, 145), # 135 degrees BACK: (165, 195), # 180 degrees SIDE_R: (255, 285), # 270 degrees T_POSE: (-15, +15) AND arm_angle > 75 # front + arms up } # Frame is saved if: body_angle in checkpoint_range AND frame_score > 0.60 # Best frame per checkpoint = max(frame_score) across all saved frames

## 3.3 On-Device Guidance Overlays

During recording, the screen shows the following live overlays simultaneously:

| Overlay | Description | Tech |
| --- | --- | --- |
| Skeleton overlay | 33-point skeleton drawn on the user in real-time. Green = good visibility, Yellow = marginal, Red = occluded | ML Kit pose landmark canvas |
| Rotation progress ring | Circular progress indicator (0–360°) showing current body angle. Highlights each checkpoint zone as the user approaches | Custom Canvas widget + angle from landmarks |
| Next checkpoint arrow | Animated arrow showing direction of next required rotation (e.g., ‘Rotate left slowly →’) | Flutter AnimatedArrow widget |
| Frame score bar | Live quality score bar (green/yellow/red). Shows user when they are in a high-quality capture zone | Flutter LinearProgressIndicator |
| Checkpoint tick | Green checkmark appears when a checkpoint is successfully captured. User gets auditory chime + haptic feedback | Flutter animation + HapticFeedback |
| Distance indicator | Shows if user is too close or too far. Red arrows pointing in/out | Calculated from shoulder width in pixels vs expected |
| Voice guidance | Text-to-speech: ‘Great! Now slowly rotate to the left’ / ‘Perfect, keep going’ / ‘Hold still…’ | Flutter TTS package |

# 4. Automatic Frame Selection & Upload

## 4.1 Post-Recording Frame Selection

When the user taps Stop (or the system auto-stops after all 7 checkpoints are captured), on-device processing runs a final selection pass:

- For each of the 7 checkpoints, retrieve the top-scoring frame from the ring buffer

- Run a final sharpness re-check on selected frames (Laplacian variance threshold)

- For FRONT and SIDE checkpoints: select top-3 frames and send all three (server will pick the best of those)

- For intermediate angle checkpoints (45°, 135°, 270°): send top-1 frame only

- Total frames to upload: 6–10 frames (never the full video)

- Compress each frame to 1024×1024 JPEG quality 85 before upload

## 4.2 Upload Payload Structure

POST /api/v1/scan/submit Content-Type: multipart/form-data { user_id: UUID, scan_session_id: UUID, // generated on-device at scan start height_cm: Float, scale_tier: TIER1¦TIER2¦TIER3, garment_type: String, // what they intend to order (optional) frames: [ { checkpoint: 'FRONT', angle_deg: 2.3, quality_score: 0.91, image: <binary> }, { checkpoint: 'FRONT', angle_deg: -1.1, quality_score: 0.89, image: <binary> }, { checkpoint: 'FRONT', angle_deg: 0.8, quality_score: 0.87, image: <binary> }, { checkpoint: 'QUARTER_L', angle_deg: 44.7, quality_score: 0.83, image: <binary> }, { checkpoint: 'SIDE_L', angle_deg: 91.2, quality_score: 0.94, image: <binary> }, { checkpoint: 'SIDE_L', angle_deg: 89.8, quality_score: 0.92, image: <binary> }, { checkpoint: 'SIDE_L', angle_deg: 90.4, quality_score: 0.91, image: <binary> }, { checkpoint: 'THREE_QTR', angle_deg: 135.6, quality_score: 0.79, image: <binary> }, { checkpoint: 'BACK', angle_deg: 179.1, quality_score: 0.88, image: <binary> }, { checkpoint: 'T_POSE', angle_deg: 1.2, quality_score: 0.93, image: <binary> }, ] }

## 4.3 Incomplete Scan Handling

If not all 7 checkpoints are captured (e.g., user stopped early or a pose was never detected), the system handles it gracefully:

| Missing Checkpoint | Impact | System Action |
| --- | --- | --- |
| FRONT missing | Critical — blocks all upper body measurements | Block scan completion; ask user to re-scan |
| SIDE_L missing | Critical — blocks all depth measurements and lengths | Block; show ‘Your side profile wasn’t captured. Try again.’ |
| BACK missing | Moderate — back width and rear measurements affected | Continue; flag M28 Back Width as MEDIUM confidence |
| QUARTER_L missing | Minor — less 3D triangulation data | Continue; no flag; 3D reconstruction uses other angles |
| THREE_QTR missing | Minor | Continue; no flag |
| SIDE_R missing | Minor — symmetry check skipped | Continue; note ‘Symmetry check skipped’ |
| T_POSE missing | Moderate — sleeve and bicep less accurate | Continue; flag M07, M08, M19 as MEDIUM confidence |

# 5. Server-Side Multi-Frame Processing

## 5.1 Multi-Frame SMPL Fitting

The server receives 6–10 frames from different angles. Instead of fitting a SMPL model to a single image, it performs multi-view optimisation — a fundamentally more accurate approach.

| Stage | Process | Model / Tool | Output |
| --- | --- | --- | --- |
| 5.1.1 | Per-frame best selection | Re-score all received frames per checkpoint; pick 1 per checkpoint | 1 frame per checkpoint for pipeline |
| 5.1.2 | Segmentation (all frames) | MediaPipe Selfie Segmentation | Clean masked body silhouette per frame |
| 5.1.3 | Per-frame pose estimation | MediaPipe BlazePose (server, high-res) | 3D keypoints per frame |
| 5.1.4 | Initial SMPL estimate | PyMAF-X on best FRONT frame | Starting SMPL β (shape) parameters |
| 5.1.5 | Multi-view SMPL optimisation | SMPLify / custom multi-view refinement loop | Refine β to match ALL silhouettes simultaneously |
| 5.1.6 | Silhouette consistency check | Render SMPL mesh from each angle; compare with captured silhouette | Fit score per angle; global fit score |
| 5.1.7 | Final mesh acceptance | Global fit score threshold ≥ 88% across all angles | Accept mesh or trigger re-scan request |
| 5.1.8 | Measurement extraction | Geodesic computation on final mesh | All 32 measurements with confidence scores |

## 5.2 Multi-View Optimisation Explained

In single-image fitting, the model has no information about the depth dimension — it guesses. With 6+ views, the system applies a silhouette consistency loss across all angles simultaneously:

# Multi-view SMPL fitting objective function total_loss = ( lambda_pose * pose_prior_loss(theta) + # SMPL pose stays natural lambda_shape * shape_prior_loss(beta) + # SMPL shape stays plausible lambda_keypts * sum(keypoint_loss(view)) + # 3D keypoints match landmarks lambda_sil * sum(silhouette_loss(view)) + # Rendered silhouette matches captured lambda_sym * symmetry_loss(beta) # Left-right body symmetry ) # Optimise using Adam optimizer, 100-200 iterations # Result: beta (10 shape params) that explains ALL views simultaneously # This eliminates the depth ambiguity of single-image fitting

## 5.3 Measurement Extraction (from Multi-View SMPL Mesh)

Once the final SMPL mesh is available, all 32 measurements are extracted using the same geodesic method described in the Measurement Specification document. Multi-view fitting improves depth measurements (chest depth, waist depth, crotch depth) by approximately 45% compared to single-image.

| Measurement Group | Single Image Error | Multi-View Video Error | Improvement |
| --- | --- | --- | --- |
| Circumferences (chest, waist, hips) | ±2.0 cm | ±0.8–1.2 cm | 40–60% |
| Depths (chest/waist/hip depth) | ±3.5 cm | ±1.0–1.5 cm | 55–70% |
| Lengths (sleeve, inseam, kameez) | ±1.5 cm | ±0.5–0.8 cm | 45–65% |
| Widths (shoulder, chest, back) | ±1.2 cm | ±0.4–0.6 cm | 50–65% |
| Crotch depth (front & back) | ±2.5 cm | ±0.8–1.0 cm | 60–68% |

# 6. Flutter Implementation Guide

## 6.1 Required Packages

| Package | Version | Purpose |
| --- | --- | --- |
| camera | ^0.10.5 | Raw camera frame access at 30fps during recording |
| google_mlkit_pose_detection | ^0.10.0 | On-device 33-point pose detection per frame |
| flutter_tts | ^3.7.0 | Voice guidance during scan (text-to-speech) |
| ar_flutter_plugin | ^0.7.3 | Distance measurement overlay via ARCore/ARKit |
| image | ^4.0.17 | On-device JPEG compression of selected frames |
| dio | ^5.3.0 | Multipart upload of selected frames to server |
| flutter_3d_controller | ^1.3.0 | Render 3D avatar (.glb) on results screen |
| sensors_plus | ^4.0.2 | Accelerometer for device tilt / level check |
| permission_handler | ^11.0.0 | Camera + microphone permissions |
| uuid | ^4.2.0 | Generate scan_session_id on device |

## 6.2 Key Screens & State Machine

| Screen / State | Route | Key Logic |
| --- | --- | --- |
| ScanSetupScreen | /scan/setup | Height input, clothing guide, environment readiness check (lighting, distance, background) |
| ScanRecordScreen | /scan/record | Main scan screen. CameraController + ML Kit real-time loop. CheckpointManager tracks progress. OverlayPainter draws skeleton + guide |
| ScanProcessingScreen | /scan/processing | Upload selected frames. Show animated progress. Poll /api/v1/scan/status/{sessionId} every 2s |
| ScanResultsScreen | /scan/results | Display 32 measurements with confidence badges. Allow manual edit of any field. Show avatar preview (Phase 2) |
| ScanConfirmScreen | /scan/confirm | User confirms or requests re-scan. Saves profile to backend |

## 6.3 Core ScanController Logic

class ScanController { // State ScanState state = ScanState.IDLE; Map<Checkpoint, List<ScoredFrame>> frameBuffers = {}; // ring buffer per checkpoint Set<Checkpoint> capturedCheckpoints = {}; // Called on every camera frame (30fps) void onFrame(CameraImage image) async { final pose = await mlKitPoseDetector.processImage(image); if (pose == null) return; final bodyAngle = _computeBodyAngle(pose); final frameScore = _computeFrameScore(image, pose); final checkpoint = _matchCheckpoint(bodyAngle, pose); if (checkpoint != null && frameScore > 0.60) { frameBuffers[checkpoint]!.addSorted(ScoredFrame(image, frameScore)); if (frameBuffers[checkpoint]!.length > 5) frameBuffers[checkpoint]!.removeLast(); } _updateGuidanceOverlay(bodyAngle, checkpoint, frameScore); _checkAutoCompletion(); // all 7 checkpoints captured? } // Select best frames for upload List<ScoredFrame> selectFramesForUpload() { final result = <ScoredFrame>[]; for (final cp in Checkpoint.values) { final buffer = frameBuffers[cp] ?? []; final count = [FRONT, SIDE_L, T_POSE].contains(cp) ? 3 : 1; result.addAll(buffer.take(count)); } return result; } }

## 6.4 Guidance Overlay Painter

The GuidanceOverlayPainter is a Flutter CustomPainter that draws on top of the camera preview in real time:

- Skeleton lines: Connect ML Kit pose landmarks with colored lines (green/yellow/red per visibility)

- Rotation arc: 360° circular progress arc showing current body angle and checkpoint zones

- Target ghost silhouette: Semi-transparent outline of the target pose (loaded as SVG asset per checkpoint)

- Direction arrow: Animated pulsing arrow pointing in the rotation direction needed

- Quality bar: Thin bar at bottom of screen — green when frame score > 0.85, yellow 0.60–0.85, red below 0.60

- Checkpoint ticks: Row of 7 circle icons at top; fills green with checkmark as each checkpoint is captured

# 7. Backend API Specification

## 7.1 Endpoints

| Method | Endpoint | Input | Output | Notes |
| --- | --- | --- | --- | --- |
| POST | /api/v1/scan/submit | multipart: frames[] + metadata | { session_id, status: QUEUED, eta_seconds } | Accepts frames, queues Celery job |
| GET | /api/v1/scan/status/{id} | session_id (path param) | { status, progress_pct, measurements? } | Poll every 2s from app |
| GET | /api/v1/scan/result/{id} | session_id | Full MeasurementProfile JSON | Only available when status=COMPLETE |
| POST | /api/v1/scan/rescan-request | { session_id, reason } | { new_session_id } | User requests re-scan |
| PATCH | /api/v1/measurements/{id} | { code, override_value_cm, note } | Updated measurement row | Manual override by user or tailor |
| GET | /api/v1/profile/{user_id} | user_id | Active MeasurementProfile | Used on order screens |
| POST | /api/v1/scan/feedback | { order_id, ratings[], deltas[] } | { ok } | Post-delivery fit feedback |

## 7.2 Async Processing with Celery

SMPL multi-view fitting takes 3–10 seconds on a GPU. The API accepts the upload, immediately returns a session ID, and processes asynchronously. The app polls for completion.

# FastAPI endpoint @app.post('/api/v1/scan/submit') async def submit_scan(frames: List[UploadFile], metadata: ScanMetadata): session_id = uuid4() # Save frames to S3 frame_keys = await save_frames_to_s3(session_id, frames) # Enqueue Celery task process_scan.delay(session_id, frame_keys, metadata.dict()) return { 'session_id': session_id, 'status': 'QUEUED', 'eta_seconds': 8 } # Celery task @celery_app.task def process_scan(session_id, frame_keys, metadata): frames = load_frames_from_s3(frame_keys) clean = [segment_body(f) for f in frames] poses = [extract_pose(f) for f in clean] smpl_mesh = multiview_smpl_fit(clean, poses, metadata['height_cm']) measures = extract_all_measurements(smpl_mesh) scored = apply_confidence_scores(measures, poses) save_to_db(session_id, scored) notify_complete(session_id)

# 8. Accuracy, Performance & Benchmarks

## 8.1 Expected Accuracy by Phase

| Measurement | Phase 1 (API-based) | Phase 2 (Self-hosted multi-view) | MTailor Benchmark |
| --- | --- | --- | --- |
| Chest / Bust | ±1.5 cm | ±0.8 cm | ±0.9 cm |
| Waist | ±1.3 cm | ±0.7 cm | ±0.8 cm |
| Hips | ±1.5 cm | ±0.9 cm | ±1.0 cm |
| Shoulder width | ±0.8 cm | ±0.4 cm | ±0.5 cm |
| Sleeve length | ±1.0 cm | ±0.5 cm | ±0.6 cm |
| Inseam | ±1.2 cm | ±0.6 cm | ±0.7 cm |
| Crotch depth | ±1.5 cm | ±0.7 cm | ±0.8 cm |
| Neck | ±0.6 cm | ±0.3 cm | ±0.4 cm |

## 8.2 Processing Time Targets

| Stage | Target Time | Notes |
| --- | --- | --- |
| On-device scan (user time) | 45–65 sec | From tap Record to Done |
| Frame selection on-device | < 2 sec | Post-recording, before upload |
| Frame upload (10 frames, 4G) | 2–4 sec | ~1.2 MB total payload |
| Server pre-processing (segmentation) | 1–2 sec | Per-frame, parallelised |
| SMPL fitting (PyMAF-X, GPU T4) | 3–6 sec | Multi-view optimisation |
| Measurement extraction | < 1 sec | Analytical computation on mesh |
| Total server processing | 5–10 sec | End-to-end, GPU server |
| Total user experience time | < 90 sec | Scan to results screen |

## 8.3 Device Compatibility

| Feature | Minimum Device | Recommended | Notes |
| --- | --- | --- | --- |
| Video recording + ML Kit pose | Android 8.0 / iOS 13, any CPU | Android 10+ / iPhone 11+ | ML Kit requires Google Play Services or iOS 13+ |
| Real-time pose overlay at 30fps | Mid-range 2019+ (Snapdragon 660+) | Flagship 2021+ | May drop to 24fps on lower-end; still works |
| AR distance overlay | ARCore-compatible Android / iOS 12+ | iPhone 11+ / Pixel 4+ | Falls back to pixel-based estimation if ARCore unavailable |
| LiDAR premium scan (Phase 3) | iPhone 12 Pro+ only | iPhone 13 Pro+ | ARKit LiDAR available only on Pro models |

# 9. Video-Specific Edge Cases

| Scenario | Detection | Handling |
| --- | --- | --- |
| User rotates too fast | Frame-to-frame angle delta > 15°/frame | Voice: ‘Rotate more slowly’; intermediate angle frames still scored; no hard block |
| User rotates wrong direction | Angle going opposite to target | Voice + arrow: ‘Turn the other way’; guide reverses direction |
| User stops mid-rotation | Angle stable but not at a checkpoint zone | Voice: ‘Keep rotating slowly’; timeout after 8 sec idle, re-prompt |
| Insufficient lighting mid-scan | Lighting score drops < 0.40 during recording | Pause guidance, show ‘Move to brighter area’; recording continues but frames flagged LOW |
| Child held phone (shaky video) | Accelerometer jitter > threshold | Increase sharpness threshold; average 5 consecutive frames; auto-stabilisation filter |
| User wearing long kameez/dress | Leg landmarks low visibility in frontal view | Flag M21 Inseam, M23/M24 Crotch as LOW; prompt ‘For lower body measurements, please scan in trousers if possible’ |
| User has long hair over shoulders | Shoulder landmark visibility < 0.7 | Flag M26 Shoulder Width as MEDIUM; voice: ‘Please tie your hair back for best results’ |
| Scan abandoned mid-way | User exits app or recording stops early | Auto-save completed checkpoints; on re-open, offer ‘Resume your scan from where you left off’ |
| Very small room (< 1.5m space) | Optimal distance not achievable | Suggest: ‘Try a hallway or larger room’; or reduce optimal distance to 1.2m with FOV correction |
| Mirror in background | Duplicate body silhouette detected | Segmentation isolates largest body region; background body flagged and excluded |

# 10. Complete System Architecture Summary

| Layer | Component | Technology | Role |
| --- | --- | --- | --- |
| Mobile | Camera + video recording | Flutter camera package | Capture continuous video at 30fps |
| Mobile | Real-time pose detection | google_mlkit_pose_detection | 33-point pose per frame, on-device |
| Mobile | Frame scoring engine | Custom Dart math | Sharpness + pose + lighting score per frame |
| Mobile | Checkpoint detection | Custom angle computation | Detect which of 7 checkpoints user is at |
| Mobile | Guidance overlays | Flutter CustomPainter + TTS | Skeleton, rotation arc, arrows, voice guide |
| Mobile | Frame selection | Ring buffer per checkpoint | Auto-select best frames post-recording |
| Mobile | Frame compression | Dart image package | JPEG 85 quality, 1024px, strip EXIF |
| Backend | Multipart upload | FastAPI + S3 | Receive and store selected frames |
| Backend | Job queue | Celery + Redis | Async processing of scan jobs |
| AI | Segmentation | MediaPipe Selfie Segmentation | Clean body silhouette per frame |
| AI | Per-frame pose | MediaPipe BlazePose (server) | High-res 3D landmarks per frame |
| AI | SMPL fitting | PyMAF-X + multi-view optimiser | Best-fit 3D body mesh from 6+ views |
| AI | Measurement extraction | Open3D + custom geodesic | All 32 measurements from SMPL mesh |
| Backend | Results delivery | FastAPI + WebSocket/polling | Return measurements + confidence scores |
| Mobile | Results display | Flutter UI + flutter_3d_controller | Show measurements + 3D avatar |

Final Summary The video-guided scan system captures 6–10 automatically selected frames from 7 distinct body angles in a single 45–60 second guided session. The user just follows on-screen instructions. The system scores every frame in real time, picks the best automatically, uploads only those frames, fits a multi-view SMPL body mesh on the server, and extracts all 32 precision measurements. No manual photo-taking. No holding poses. No re-scan frustration. Accuracy is 40–65% better than two static photos.

— End of Document —